BIPM GPS calibration information sheet

#### ANNEX 2

| Laboratory: |  |  |  |  |  | NICT TOKYO JAPAN |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date and hour of the beginning of measurements: |  |  |  |  |  | # 17 May 2004 (MJD 53142) UTC:01hxxmxxs |  |  |  |
| Date and hour of the end of measurements: |  |  |  |  |  | # 24 May 2004 (MJD 53149) UTC:01h30mxxs |  |  |  |
| Receiver setup information |  |  |  |  |  |  |  |  |  |
|  |  | Local: TTR6 |  |  |  |  | Local:E-80 |  | Portable: NML |
|  Maker: |  | AOA |  |  |  |  | Javad |  | NML/Topcon |
|  Type: |  | TTR-6 |  |  |  |  | Euro-80 |  | Euro-80 Dual Frequency |
|  Serial number: |  | 451 |  |  |  |  | 8PN45EETDKW |  | 8R633IOLON4 |
|  Receiver internal delay (GPS) : |  | 44.8ns |  |  |  |  | 47.2ns |  | 44.79ns |
|  Receiver internal delay (GLO) : |  | - |  |  |  |  | - |  | - |
|  Antenna cable identification: |  | TTR6(219.6ns) |  |  |  |  | E80 |  | NML IF |
| Corresponding cable delay : |  | 250.0ns |  |  |  |  | 152.15ns |  | (159.8 ± 1.0) ns |
|  UTC cable identification: |  | GPS G |  |  |  |  | UTC(NICT)1pps C3 |  | UTC(NICT)1pps JRC#2 |
| Corresponding cable delay : |  |  |  |  |  |  |  |  |  |
| Delay to local UTC : | Header Value | 316.1ns |  |  |  |  | 344.123ns |  | 85.64ns |
|  | Meas. Value | 306.43ns |  |  |  |  | 344.123ns |  | 319.97ns |
|  Receiver trigger level: |  | 0.5V |  |  |  |  | 0.4V |  | 0.5 V |
|  Coordinates reference frame: |  | WGS-84 |  |  |  |  | WGS-84 |  | WGS-84 |
| Latitude or X m |  | -3942161.90m |  |  |  |  | -3942164.215m |  | -4648204.271m |
| Longitude or Y m |  | 3368284.20m |  |  |  |  | 3368281.976m |  | +2560477.026m |
| Height or Z m |  | 3701886.69m |  |  |  |  | 3701887.149m |  | -3526504.952m |
| Antenna information |  |  |  |  |  |  |  |  |  |
|  |  | Local: TTR6 |  |  |  |  | Local:E80 |  | Portable: |
|  Maker: |  | AOA |  |  |  |  | Javad |  | Topcon/Japad |
|  Type: |  |  |  |  |  |  | RegAnt 1, |  | MarAnt |
|  Serial number: |  | Down Converter S/N449 |  |  |  |  | S/N RA0238 |  | MAGGD #0191 |
| ## If the antenna is temperature stabilised |  |  |  |  |  |  |  |  |  |
|  Set temperature value : |  |  |  |  |  |  |  |  | - |
| Local antenna cable information |  |  |  |  |  |  |  |  |  |
|  Maker: |  |  |  |  |  |  | Times Microwave-systems |  | NML IF |
|  Type: |  | RG58AU |  |  |  |  | LMR-400 DB |  |  |
|  Is it a phase stabilised cable: |  | No |  | No |  |  | No |  |  |
|  Length of cable outside the building : |  | Approx. 18 m |  |  |  |  | Approx. 18 m |  | Approx. 18 m |
| General information |  |  |  |  |  |  |  |  |  |
|  Rise time of the local UTC pulse: |  |  |  |  | 4.7ns(10%-90%)pulse height 4.59v DC |  |  |  |  |
| #  Is the laboratory air conditioned: |  |  |  |  | YES |  |  |  |  |
|  Set temperature value and uncertainty : |  |  |  |  | GPS RX Room 23℃±2℃ |  |  |  |  |
|  Set humidity value and uncertainty : |  |  |  |  | N/A |  |  |  |  |
| Cable delay control |  |  |  |  |  |  |  |  |  |
| Cable identification |  |  | delay measured by NML |  |  |  |  | Delay measured by local method |  |
| NML-IF Antenna cable |  |  | (159.8 ± 1.0) ns |  |  |  |  |  |  |

Plot of the experiment set-up: ## Link to the local UTC of both receivers and Antenna positions ANT TTR6 GPS RX HP5087A 5MHz Amp. Oscilloquartz5MHz Amp. Oscilloquartz5MHz Amp. Sigmatau AOG model 110 Oscilloquartz5MHz Amp. CS Clock 5071A 5MHz Pulse Dist. Amp. Oscilloquartz10MHz Amp. OscilloquartzMultiplier x2 HP5087A 10MHz Amp. ANT Euro-80 MarAnt NML/Topcon TRAK 6487 1PPS Amp. TRAK 6487 1PPS Amp. TRAK 6487 1PPS Amp. SR620. 5MHz UTC(NICT) 1PPS 5MHz 10MHz 5MHz 1PPS 1PPS 1PPS 10MHz Counter Rec. Out 1PPS 10MHz Ext. Counter Ref. Clock Room Room 302 Room 305